Right-click on Connections, in the Workspace window in DaqFactory, to create a new connection. Enter an arbitrary name, then enter the IP address from the list below; passwords are not required (the connection name can be anything that makes sense to you; the IP address fully specifies the remote computer).  Verify that “Full Stream” is NOT checked (this will reduce the amount of traffic on the network, and will only display channels deliberately broadcasted from each instrument)

ConnectionName	IP address		ChannelName			Conversion		Units		PI

NOy			10.172.240.49		NO_counts			(x-800)/5.5		ppbv		Tom

NOy_counts			(x-800)/5.5		ppbv

O3_counts			(x-20)/2.5		ppbv

CO2			10.172.240.26		CO2_mV_serial		(x/0.3)			ppmv		Tom/Jeff

CO			10.172.240.40		C00_pmt0						ppbv		John H.

SO2			10.172.240.42		SO2_C01_fluor_int					ppbv		John H.

LPAS			10.172.240.65		ethylene						ppbv		Joost/Carsten

NMASS		10.172.240.34		counter1			(x1013/(90*4.5) 	#/cc		Chuck

WLOPC		10.172.240.62		vcourse						um3/cc		Adam

FltSci			10.172.240.47		AOC_GPSLat						degrees	Ken

(subset shown)	AOC_GPSLon					degrees

AOC_GPSalt						meters

AOC_Palt						meters

AOC_Ralt						meters

AOC_AmbTemp					°C

AOC_DewPtTemp					°C

AOC_H2Omr						g/kg

AOC_StaticPrs					millibar

AOC_TDLH2O					°C (?)

AOC_WindDir					degrees

AOC_WindSpd					m/s

AOC_CabinPrs					millibar

AOC_GndSpd						m/s

AOC_Heading						degrees

ConnectionName	IP address		ChannelName			Conversion		Units		PI

FltSci	(cont.)		10.172.240.47		AOC_DryExtinction					Mm-1		Ken

AOC_PAN						ppbv

AOC_CalPAN					ppbv

AOC_N2O5						ppbv

AOC_WASCanNumber				#

AOC_WASMotorCurrent				amps

AOC_WASManifoldP				psig (?)

CIMS			10.172.240.54		NH3_cntl1						Hz		John N.

Use “ConnectionName.ChannelName” to call these remote channels in your copy of DF.  For example, to display Chuck’s small particle data, add a trace called “NMASS.counter1” to your graph.

Note that it sometimes takes several attempts to successfully connect to the remote copy of DF.  Left-click on the connection name in the Workspace window, and click on “reset connection” until the Packets Received counter begins to increment.